🚀 Multi-Cluster IoT Routing System - Complete Project Documentation

Project Overview

This project implements a hierarchical cluster-based routing system for IoT sensor networks using Cisco Packet Tracer. The system consists of 3 clusters, each with 2 member nodes and 1 cluster head, all communicating with a central server.

📋 Table of Contents

- Network Architecture

- Hardware Requirements

- IP Address Scheme

- Physical Topology Setup

- Network Configuration

- Code Implementation

- Testing & Verification

- Troubleshooting

🏗️ Network Architecture

System Design

┌─────────────────┐

│   Server-PT     │

│  192.168.1.1    │

│   (Port 5000)   │

└────────┬────────┘

│

┌────────┴────────┐

│  AccessPoint-PT │

│    (WSN_AP)     │

└────────┬────────┘

│

┌───────────────────┼───────────────────┐

│                   │                   │

┌────┴────┐         ┌────┴────┐        ┌────┴────┐

│  CH-1   │         │  CH-2   │        │  CH-3   │

│ .1.100  │         │ .1.110  │        │ .1.120  │

└────┬────┘         └────┬────┘        └────┬────┘

│                   │                   │

┌────┼────┐         ┌────┼────┐        ┌────┼────┐

│    │    │         │    │    │        │    │    │

┌─┴─┐┌─┴─┐        ┌─┴─┐┌─┴─┐      ┌─┴─┐┌─┴─┐

│M1-1││M1-2│        │M2-1││M2-2│      │M3-1││M3-2│

│.101││.102│        │.111││.112│      │.121││.122│

└───┘└───┘        └───┘└───┘      └───┘└───┘

│     │            │     │          │     │

[S]   [S]          [S]   [S]        [S]   [S]

Legend:

- CH: Cluster Head (receives from members, aggregates, forwards to server)

- M: Member Node (reads sensor, sends to cluster head)

- [S]: Motion Sensor

- Numbers: Last octet of IP address (192.168.1.X)

🔧 Hardware Requirements

Devices Needed in Packet Tracer:

| Device Type | Quantity | Device Name | Purpose |
| --- | --- | --- | --- |
| Server-PT | 1 | Server-PT BaseStation | Central data collection server |
| AccessPoint-PT | 1 | WSN_AP | Wireless network hub |
| MCU-PT | 9 | ClusterHead-1/2/3, Member1-1/2, Member2-1/2, Member3-1/2 | IoT microcontrollers |
| Motion Sensor | 6 | Sensor1-1 through Sensor3-2 | PIR motion detection sensors |

Connections Required:

- Server ↔ AccessPoint: Ethernet Cable (Copper Straight-Through)

- AccessPoint ↔ MCUs: Wireless (802.15.4)

- MCU ↔ Sensors: IoT Custom Cable

🌐 IP Address Scheme

Complete IP Allocation Table

| Device | IP Address | Subnet Mask | Default Gateway | Port | Role |
| --- | --- | --- | --- | --- | --- |
| Server-PT | 192.168.1.1 | 255.255.255.0 | - | 5000 | Data Collection Server |
| AccessPoint | 192.168.1.254 | 255.255.255.0 | 192.168.1.1 | - | Wireless Hub |
| Cluster 1 |  |  |  |  |  |
| ClusterHead-1 | 192.168.1.100 | 255.255.255.0 | 192.168.1.1 | 5001 | Cluster 1 Head |
| Member1-1 | 192.168.1.101 | 255.255.255.0 | 192.168.1.1 | 5011 | Cluster 1 Member |
| Member1-2 | 192.168.1.102 | 255.255.255.0 | 192.168.1.1 | 5012 | Cluster 1 Member |
| Cluster 2 |  |  |  |  |  |
| ClusterHead-2 | 192.168.1.110 | 255.255.255.0 | 192.168.1.1 | 5002 | Cluster 2 Head |
| Member2-1 | 192.168.1.111 | 255.255.255.0 | 192.168.1.1 | 5021 | Cluster 2 Member |
| Member2-2 | 192.168.1.112 | 255.255.255.0 | 192.168.1.1 | 5022 | Cluster 2 Member |
| Cluster 3 |  |  |  |  |  |
| ClusterHead-3 | 192.168.1.120 | 255.255.255.0 | 192.168.1.1 | 5003 | Cluster 3 Head |
| Member3-1 | 192.168.1.121 | 255.255.255.0 | 192.168.1.1 | 5031 | Cluster 3 Member |
| Member3-2 | 192.168.1.122 | 255.255.255.0 | 192.168.1.1 | 5032 | Cluster 3 Member |

🔨 Physical Topology Setup

Step-by-Step Device Placement

Step 1: Add Server

- From device list, drag Server-PT to workspace

- Label it: Server-PT BaseStation

- Position it at the top-center of your workspace

Step 2: Add Access Point

- Drag AccessPoint-PT below the server

- Label it: WSN_AP

- Connect Server to AccessPoint:

  - Click Copper Straight-Through cable

  - Server: FastEthernet0 → AccessPoint: Ethernet0

Step 3: Add Cluster 1 Devices

- Drag 3 × MCU-PT to left side of workspace

- Label them:

  - ClusterHead-1

  - Member1-1

  - Member1-2

- Drag 2 × Motion Sensor

- Label them:

  - Sensor1-1

  - Sensor1-2

- Connect sensors to members:

  - Click IoT Custom Cable

  - Sensor1-1 → Member1-1 (D0 pin)

  - Sensor1-2 → Member1-2 (D0 pin)

Step 4: Add Cluster 2 Devices

- Drag 3 × MCU-PT to middle of workspace

- Label them:

  - ClusterHead-2

  - Member2-1

  - Member2-2

- Drag 2 × Motion Sensor

- Label them:

  - Sensor2-1

  - Sensor2-2

- Connect sensors to members (D0 pin)

Step 5: Add Cluster 3 Devices

- Drag 3 × MCU-PT to right side of workspace

- Label them:

  - ClusterHead-3

  - Member3-1

  - Member3-2

- Drag 2 × Motion Sensor

- Label them:

  - Sensor3-1

  - Sensor3-2

- Connect sensors to members (D0 pin)

Step 6: Verify Wireless Connections

- All 9 MCUs should show wireless connection lines to the AccessPoint

- If not visible, MCUs might not be in range - move them closer

⚙️ Network Configuration

CRITICAL: Disable DHCP First!

On Server:

- Click Server-PT BaseStation

- Go to Services tab

- Click DHCP

- Turn OFF DHCP Service

- This prevents automatic IP assignment

Configure Server

- Click Server-PT BaseStation

- Go to Config tab

- Click FastEthernet0

- Set:

  - IP Address: 192.168.1.1

  - Subnet Mask: 255.255.255.0

- Close

Configure Access Point

- Click AccessPoint-PT (WSN_AP)

- Go to Config tab

- Click FastEthernet0 or Port 1

- Set:

  - IP Address: 192.168.1.254

  - Subnet Mask: 255.255.255.0

  - Default Gateway: 192.168.1.1

- Verify Wireless settings:

  - SSID: Default (or set to "WSN_Network")

  - Mode: Access Point

- Close

Configure All MCUs (9 devices)

For each MCU, follow this pattern:

Cluster 1:

ClusterHead-1:

- Click ClusterHead-1

- Config → Wireless0 (or FastEthernet0)

- IP Configuration: Static

- Set:

  - IP: 192.168.1.100

  - Subnet: 255.255.255.0

  - Gateway: 192.168.1.1

Member1-1:

- IP: 192.168.1.101

- Subnet: 255.255.255.0

- Gateway: 192.168.1.1

Member1-2:

- IP: 192.168.1.102

- Subnet: 255.255.255.0

- Gateway: 192.168.1.1

Cluster 2:

ClusterHead-2:

- IP: 192.168.1.110

- Subnet: 255.255.255.0

- Gateway: 192.168.1.1

Member2-1:

- IP: 192.168.1.111

- Subnet: 255.255.255.0

- Gateway: 192.168.1.1

Member2-2:

- IP: 192.168.1.112

- Subnet: 255.255.255.0

- Gateway: 192.168.1.1

Cluster 3:

ClusterHead-3:

- IP: 192.168.1.120

- Subnet: 255.255.255.0

- Gateway: 192.168.1.1

Member3-1:

- IP: 192.168.1.121

- Subnet: 255.255.255.0

- Gateway: 192.168.1.1

Member3-2:

- IP: 192.168.1.122

- Subnet: 255.255.255.0

- Gateway: 192.168.1.1

💻 Code Implementation

Now I'll create all the code for your devices: